Hardware Assembly Instructions:

- Take everything out of boxes

Figure 1: Top of tripod

Figure 1: Top of tripod

Remove and categorize all parts needed for assembly, in the same step, check for any instruction manuals that may come with the individual parts

- Assemble tripod

  - See Figure 1 and Figure 2

- Screw data logger into the enclosure

Figure 2: Bottom of tripod

Figure 2: Bottom of tripod

Place plastic grommets into holes in the correct spot

  - Attach with screws

- Screw battery holder into enclosure

  - Place screw through battery holder and white backing of enclosure

Figure 3: Battery holder attached to enclosure

Figure 3: Battery holder attached to enclosure

Secure with bolt on the back of the white backing of enclosure

  - See Figure 3

- Connect the data logger to the green cord from the enclosure

  - Unscrew the small screw

Figure 4: Wire attached to datalogger

Figure 4: Wire attached to datalogger

Strip off the end of the green wire

  - Place wire into hole of data logger

  - Rescrew screw into the hole

  - See Figure 4

- Connect wire from lufft to datalogger

Figure 5: Close up of wires

Figure 5: Close up of wires

Place each wire from lufft into the correct place in the datalogger

  - Green = G

  - Yellow = C1

  - White = -

  - Brown = red +

  - Tape up the rest of the colored wires with electrical tape

  - See Figure 5 and Figure 6

Figure 6: Close up of wires

Connect battery to data logger

  - Need new wire connectors and a red and a black wire

  - Plug red wire into + BAT

  - Plug black wire into - BAT 12VAC

  - Secure new connectors to other end of the wire

  - Plug black wire into black terminal on battery

  - Plug red wire into red terminal on battery

Figure 7: Battery connected to datalogger

Figure 7: Battery connected to datalogger

Zip tie red and black wires together to clean up enclosure

  - See Figure 7

- Cut off end of wire on the cord from the solar panel

  - Strip down ends of cord into 2 separate wires

  - See Figure 8

- Attach compression coupling to top of metal pole

Figure 8: Cut end of wire from solar panel

- Mount housing to tripod using a 1 ¾ inch saddle clamp and reinforcement plate

Equipment to Purchase:

- Wire (For battery)

- Screws (For securing battery holder)

- Bolts (For securing battery holder)

Figure 9: Housing mounted on tripod

Female disconnects for wires (To connect wire to battery)

- 1 inch compression coupling

- 1.5-inch cap

- 1 ¾ inch saddle clamps (2)

- Reinforcement plate (2)